Air Conditioner (AC) Prep and Install

Tools needed – 8mm wrench (or 5/16”) and socket (socket set), Milwaukee multi-screwdriver, scissors to cut strapping, white rolling cart

Prepping the AC Unit

- Cut strapping on the AC box. Pull up on the top of the box leaving the AC sitting on the box bottom

- Check all corners, control panel, etc. for shipping damage. If damaged, reinstall box top, tape shut and leave a note on blue tape of the damage. The AC will be returned to Lowes for replacement.

- If the AC unit is in good condition, lift the unit and place on a white rolling cart.

- Remove shipping tapes from the AC

- Plug in the AC and make sure it is operating. If not operating, put back into the box for return/replacement from Lowes

- Remove all the installation pieces from the box. Not needed – accordion side pieces, white metal bracket, white foam boards, install and operating manuals, bag of parts that has the remote batteries

- Set aside the remote and black foam pieces

- Needed to install – R and L triangle brackets, package of screws (not battery bag)

- Remove the Phillips screw near the front, lower left side (see photo), set aside

- Open the front, white cover by pulling on the top sides. Lift and remove the cover.

- Remove the air filter and set aside with the white cover

- Remove the 2 Phillips screws that hold the black front cover.(see photo) The screws are about 2” from the top on either side, set screws aside

- Pull to either side the black plastic cover at about 4” from the bottom. This will release the clips holding in the lower part of the cover.

- Once the lower clips are released, lift the black cover to release the top clips. The cover will remain attached by the wiring.

- Fold the black cover over so it rests on the top.

- Position the AC on the cart so it can be tipped up on the front, turn the AC so it is resting on the front being careful with the power cord and front cover wiring.

- Once the AC is on the front, pull up on the outer shell of the AC. You may need to wiggle the outer cover to get it to lift up. Set aside the outer shell.

- Turn the inner AC back on its bottom. Reinstall the black cover using the 2 screws.(see photo) Install the lower left side screw.(see photo)

- Install the air filter and white cover.

- Place the inner AC part back in the bottom packaging. Replace box top and set unit aside to be installed just before the home is ready to leave the Hope Factory

- Turn the outer part of the AC upside down. Install the R and L triangle brackets to the bottom of the unit using the 4 – 5/16” (or 8mm) bolts and nuts. Insert the 5/16” bolts from inside the cover so the nut is installed on the bracket side. The front hole of the bracket should line up with the second hole in the cover closest to the front (see photo). The back hole in the bracket aligns with another hole in the AC cover. If installed with the nut inside the cover, the inner AC unit won’t slide in later. Tighten the nuts securely.

- Using a 3/16” bit and drill motor, drill two holes in the outer part of the AC on the top and bottom (see photos).

Installing the AC Outer Unit, after the Interior Plywood is Installed and Both Inner and Exterior Holes are Cut Open

Tools needed - impact driver with ¼” hex driver bit (station 1) and Torx bit

Supplies – 6 Simpson screws (station 1), 4 - 1-5/8” tan Torx screws

- Verify that the interior plywood and exterior siding are flush with the 2 by 4 frame opening. Trim as needed.

- Slide the AC outer part into the back of the home, one person inside to support it and another outside to install the screws into the brackets

- Adjust side to side as needed. The front, bottom lip of the AC cover rests on the 2 by 4 framing.

- Install 2 Simpson screws in the upper holes of the support brackets; one on each side. Install 4 Simpson screws in each bracket on the lower holes, 2 on each side.

- Install two 1-5/8” tan screws through the bottom of the AC outer shell into the 2 by 4 framing. (see photo)

- Shim and install two 1-5/8” tan screws through the top of the AC outer shell into the 2 by 4 framing (see photo). Make sure the outer top part of the AC isn’t bent by the screws

- Install larger foam weather stripping that comes with the AC along the bottom of the box.

- This completes the installation of the AC outer unit. The exterior and interior trim can then be installed.

To install the AC inner unit

Tools – Phillips screwdriver

- Move the AC inner unit inside the tiny home. Remove the cardboard box top and take the inner unit out of the box bottom.

- Open the white front cover, lift, remove and set aside along with the air filter.

- Remove the lower left side screw (see photo).

- Remove the two screws holding the black plastic cover. (see photo)

- Insert and slide the AC inner unit into the preinstalled outer cover being careful of the attached wiring to the black cover. The inner AC unit should slide fully into the cover so the holes on the lower left side line up. Install the single screw on the lower left side.(see photo) This is all that holds the inner and outer parts together.

- Install the black plastic cover by aligning the top clips and then aligning the lower right and left side clips. Tap the side clips to fully seat.

- Install the two screws in the black cover

- Install the air filter and white cover (insert bottom pins first and push in the top of the cover)

- Plug in the AC and verify it is working