BACK SAW CUT LIST: CUT FOR 2 HOMES

Platform Frame Cart:

| 24 | 2x6x93 in | Use 8’ 2x6 |
| --- | --- | --- |
| 8 | 2x6x144 in | Use 12’ 2x6 |

Back Saw Cart:

| Quantity | Length | Notes |
| --- | --- | --- |
| 4 | 89 in | Use 8’ 2x4 |
| 32 | 84 in | Use 8’ 2x4 |
| 4 | 37 ¾ in | Take 20½” scrap to front saw cart |
| 8 | 24 in | Scrap, or use another 8’ 2x4 |

Bird Block: (take to Band Saw for 18.5° angle cut)

| 16 | 2x4x22¼ in | Use 8’ 2x4 |
| --- | --- | --- |
| 8 | 2x4x18 in | Use 8’ 2x4 |

Rafters:

| 44 | 56 ¾ in, 18.5° cut one end | Use 10’ 2x4, for 2 pieces |
| --- | --- | --- |

Mark rafter cutout using pattern, then take to Band Saw.