FRONT SAW CUT LIST: CUT FOR 2 HOMES

On a single lumber cart, cut these lengths of 2x4

| 4 | 2x4x89 in | Use 8’ 2x4 |
| --- | --- | --- |
| 30 | 2x4x84 in | Use 8’ 2x4 |
| 8 | 2x4x24 in | Use 8’ 2x4 |
| 2 | 2x4x13¾ in | Use 8’ 2x4, or scrap |
| 4 | 2x4x20½ in | Use block from Back Saw, or scrap |

1x4 TRIM -- SHIPMENT

From 12’ material, cut parts in this order

| Length | Use | Notes |
| --- | --- | --- |
| 93½” | Corner Board | Save cutoff pieces, in 4 equal stacks |
| 39½” | Window Side | Use 2 stacks of cutoffs |
| 31½” | Window Top | Use 1 stack of cutoffs; bevel top at 30˚ |
|  | Window Bottom | Leave 4th stack uncut |