Installing a door – checklist (rtfm)

Prepare:

- Reinforce door trim corners – brown screws

- Be sure to remove plastic feet from bottom of door frame

- Flexible flashing across bottom of opening and 8” up each side (see instruction manual)

Install:

- Tilt door into opening, bottom first.  (Remove plug holding the door shut first.)

- Shims to align door frame with steel door

  - Parallel across top of door

  - Parallel on latch side of door

- # 10 Screws to anchor frame to building.  Shim behind door frame at each location:

  - above top hinge

  - high on latch side of frame

  - above bottom hinge

  - low on latch side of frame

  - above middle hinge

  - two screws, above and below latch cutout.  See instruction book for positioning.  Aim for 1/8” gap between door and frame on this side.

  - two screws up through top of door frame.  Use plywood to fill most of the opening.

- Three screws through flooring into platform.  Use 3” plated screws to match ADA threshold

Finish:

- Remove one screw from each hinge, and replace with 3” plated screw.

- Trim visible flexible flashing from either side of exterior door trim and from under threshold.

- Trim all shims to be even with interior plywood walls

- Remove weatherstripping from sides and top of door frame, and save these.

- If wiper comes off the bottom of the door, remove it completely, and save with other weatherstripping.