INSULATION CUT LIST

INTERIOR INSULATION

Cut and load in this order, back to front of the insulation cart, bottom level.

(12)	24 in x 84 in		Mark “Side Wall”

(2)	24 in x 42 in		Mark “Under Window” (2 pieces from one bat)

(1)	24 in x 84 in		Mark “Back Wall”

(1)	24 in x 67½  in	Mark “Under A/C”

(2)	15 in x 84 in		Mark “Skinny”

EXTERIOR INSULATION

Cut and load in this order, back to front of the insulation cart, top level.

(8)	24 in x 45 ½ in	Mark “Middle Roof” (2 pieces from one bat)

(4)	18 in x 45 ½ in	Mark “End Roof” (2 pieces from one bat)

Save some of the 12 in cutoff pieces for use as insulation for structural triangles; small pieces should be put into trash.  There is a black plastic bag for the saved insulation, next to the insulation storage area.