Interior Panels Cut List

This cut list will produce paneling for two homes.  Get 3 orange carts for this work.

Horizontal Cuts

Set up the panel saw for horizontal cuts.  Using the templates, cut in this order and stack on a temporary cart:

| Quantity | Height | Width |
| --- | --- | --- |
| 6 | 43 ½” | 96” |
| 4 | 44” | 96” |
| 7 | 38 ¼” | 96” |

Vertical Cuts

Set up the panel saw for vertical cuts.  For each home, cut the following panels and mark each with the panel designator letter, and indicate the cut edge.  Stack these panels on one cart per home in this order (back to front):

| Width | Length | Label | Source |
| --- | --- | --- | --- |
| 48” | 88” | D | Uncut panel from plywood stack |
| 38 ¼” | 46 ¼” | E | Cut two E panels from one 38¼ x 96 |
| 48” | 88” | G | Uncut panel from plywood stack |
| 38 ¼” | 88” | H | Temporary cart (thinnest panels) |
| 44” | 86 ½” | C | Temporary cart (widest panels) |
| 48” | 92” | A | Uncut panel from plywood stack |
| 38 ¼” | 92 | B | Temporary cart (thinnest panels) |
| 44” | 86 ½” | C | Temporary cart (widest panels) |
| 48” | 92” | A | Uncut panel from plywood stack |
| 38 ¼” | 92” | B | Temporary cart (thinnest panels) |
| 43 ½” | 44” | R2 | Temporary cart (middle width panels) |
| 43 ½” | 44” | R2 | Remainder from previous R2 |
| 43 ½” | 92” | R1 | Temporary cart (middle width panels) |
| 43 ½” | 92” | R1 | Temporary cart (middle width panels) |