STRUCTURAL TRIANGLE PARTS

CUT FOR 6 HOMES (12 triangles)

Cut these (rough) lengths of 2x4, and place them on a yellow cart.

| 12 | B | 89 5/8” | Use 8’ 2x4 |
| --- | --- | --- | --- |
| 12 | A | 96” (no cut needed) | Use 8’ 2x4 |

Using the green chop saw and jig (set up for 18.5° angle cuts), trim both ends of these.  The longer one is used below for A pieces

Set the (red) back chop saw to an 18.5° angle, and trim the following, setting each 2x4 on the saw table standing on its shortest edge:

| 24 | A | 41” | Cut two pieces from each blank, being sure that the long point previously cut is against the back rail and even with the stop |
| --- | --- | --- | --- |
| 24 | D | 6-3/16” | From scraps or 12” scrap blocks |
| 24 | E | 11-9/16” | From 12” blocks |

Set the back chop saw back to 0°. Cut

| 12 | C | 11-5/16” | Use 12” scrap blocks |
| --- | --- | --- | --- |
| 24 | E | 11-5/16” | Cut the tip off the slanted end of each piece |